Priorities:

1: High

2: Medium

3: Low

Machine setup:

Motors are arranged in 15 x 20 matrix with an average spacing of 100 mm distance. Motors are connecting to a leadscrew of 1 meter length.

Motors are connected in a daisy chain setup.

Power will be distributed through power centers if necessary to avoid long cable lengths and easy setup and fault finding.

Software should be run inside Grasshopper for Rhino3D if possible.

Status check of motors (1)

All motors are on and reading no errors.

Read position

Read temperature

Read humidity

Read motor ids

Save

Save last position

Save last activated time and date

Save last temperature

Save last humidity

Calibrate (1)

When program start up motors need to be calibrated.

If motor last position is not equal to calibration csv file, then calibrate.

This happens by having the motor rotate towards the bottom of leadscrew where it will get resistance from hitting the bottom and draw current spike. When current spikes (stall) the motor will be in pre-calibration state. Each motor will have measured offset (calibrations csv) from pre-calibration state/position to calibrated position, for example 2 rotations.

Its important to find the right spike of current to use for calibration setting. This has to be measure and tested.

When program is running, and shaping has finished

Shape (1)

Motors are position in a matrix of for example 20 x 10. Each motor will receive a unique position/speed and a speed derived from a csv file. Positions will be calculated from Rhino 3D from an offset of a curved surface.

The function should have a start and stop function.

If motor last position is equal to calibrations csv file, then shape function is possible.

Read csv shape file and transfer values

After final position is reached send validation

Between two shapes (3)

Addition to Shape functions will be to move between two different csv files.

Over stretch and under stretch in percentages (2)

Percentage of 1-15% that motors position can offset positive or negative

Position and time for shape and max distance and actual distance (2)

Read out for current position of motors

Read out for current position of motors

Read out for current position of motors

Current readout while running (2)

Readout continuously on what each motor is drawing of current.

Position relative to neighbor (3)

Check individual motor position up against neighbor by checking for max allowable slope or angle between motors in matrix.

Laser output (3)

Feedback (3)

Enable each motor to reposition individually to a measured position.

Feedback can be read as force feedback or positional feedback