# FIRMWARE UPLOAD GUIDELINE

## TOOLS REQUIRED

- J Link Debug Probe

- EtherCAT Interface Board and Control Board

- J link Command software (The link is for 64-bit Windows)

## STEP BY STEP INSTRUCTIONS

- Download and install J link documentation pack. Inside the pack, J link commander will also be installed on the system.

- Once the software is installed, proceed to make the hardware connections between the EtherCAT Boards and the J link Debug Probe as described below

Table 1 Pinout connections between Jlink Probe and EtherCAT board

| S.No | J Link Debug Probe Pin | EtherCAT Board Pin |
| --- | --- | --- |
| 1 | VTref | VCC_3V3 |
| 2 | SWDIO | SWDIO |
| 3 | SWCLK | SWDCLK |
| 4 | SWO (Optional) | SWO (Optional) |
| 5 | RESET | RST |
| 6 | GND | GND |

- Images are also shared from both the debug probe and the board for easiness

Figure 1 JLink Probe pinout information

Figure 2 EtherCAT Board pinout information

- Once the connections are established, power up the PCB board using the headers on Interface Board with an adequate power supply.

- Connect the J link to the target PC and then afterwards open Jlink commander software.

- Verify VTref are being read as 3.3V to make sure the board is properly connected to the Jlink Probe as shown in Figure 3

Figure 3 Screenshot showing VTref for unconnected Jlink

- Type connect on the command line interface to establish connection with the custom EtherCAT board.

- Once the connection is established, an interface may open where it will ask for specific MCU part number, proceed to select ATSAME53J18 from the list and click OK.

- Another prompt will ask for type of Interface? (SWD, JTAG etc). Type S and press Enter to proceed.

- Choose default for the rest of the options by pressing Enter.

- Once finished, type loadfile <path\to\hex\file>

- Press Enter and the program will start to load. Once the program has loaded successfully, an On-screen message will indicate success.

- Disconnect the Jlink Probe and power cycle the board (turn the power OFF / ON) and start testing!