J-LINK EDU Mini SMD Adapter

(https://vi.aliexpress.com/item/1005004708738854.html?src=google&gatewayAdapt=glo2vnm)

- 10pin 1.27mm FFC Cable	(Will connect with J-Link EDU Mini)

- 20pin 2.54mm FFC Cable	(Will connect with our controller board)

Make these connections between Adaptor and controller board;

| Pin function | Header on adaptor | Header on controller board |
| --- | --- | --- |
| VRef | Pin 1 | Pin 1 |
| SWDIO | Pin 7 | Pin 2 |
| SWCLK | Pin 9 | Pin 4 |
| SWO | Pin 13 | Pin 6 |
| RESET | Pin 15 | Pin 10 |
| GND* | Pin 4, 6, 8, 10, 12 | Pin 3, 5, 9 |

* Connecting only one pin between the two headers is sufficient